| ENGINE | R110 | R135 |
| --- | --- | --- |
| Battery type | Z.E. 50 |  |
| DRIVING RANGE |  |  |
| Average range in real life condition summer / winter (km) | In the process of being approved |  |
| homologation protocol | WLTP(1) |  |
| homologated range in combined cycle (km)(2) | 390 | 390 |
| homologated consumption in combined cycle (Wh/km) | In the process of being approved | In the process of being approved |
| wheel size of the homologated version(4) | 15’’ |  |
| ENGINE |  |  |
| Electric engine technology | Synchronous with rotor coil |  |
| Peak power kW EEC (h.p.) / at peak power rating (rpm) | 80 (108) / 3,395 à 10,886 | 100 (135) / 4,200 à 11,163 |
| Peak torque Nm EEC (Nm) / at peak power rating (rpm) | 225 / 500 à 3,395 | 245 / 1,500 à 3,600 |
| BATTERY |  |  |
| Usable capacity (kWh) | 52 |  |
| Technology | Lithium-ion |  |
| Total voltage (volts) | 400 |  |
| Number of modules/cells | 12 / 192 |  |
| Battery weight (kg) | 326 |  |
| CHARGE TIME(3) |  | " |
| Charger | Adaptable for single or three-phase power AC « from 2 kW to 22 kW » and DC |  |
| Home socket (10A) | 34 hours and 30 minutes |  |
| Green-Up socket / 3,7kW Wallbox (single phase 16A) | 16 hours and 10 minutes |  |
| 7.4 kW (32 A single-phase charging point) | 9 hours and 25 minutes |  |
| 11 kW (16 A three-phase charging point) | 6 hours |  |
| 22 kW (32 A three-phase charging point) | 3 hours |  |
| 43 kW (63 A three-phase charging point) (80% battery charge) | 3 hours |  |
| DC 50 kW quick charge (0-80%) | 1 hour and 10 minutes |  |
| Standard wheel rims (") | 15’’ (Life) – 16’’ (Life, Zen) – 17’’ (Zen) | 16’’ (Zen, Intens) – 17’’ (Zen, Intens) |
| Tyre type and dimensions | Michelin Primacy 4 15’’: 185/65 R15 – 16’’: 195/55 R16 – 17’’: 215/45 R17 (front), 205/45 R17 (rear) | Michelins Primacy 4 16’’: 195/55 R16 – 17’’: 215/45 R17 (front), 205/45 R17 (rear) |

| ENGINE | R110 | R135 |
| --- | --- | --- |
| Battery type | Z.E. 50 |  |
| GEARBOX |  |  |
| Gearbox type | Gearbox with single-speed reduction gear bar |  |
| Number of forward gears | 1 |  |
| PERFORMANCE |  |  |
| Top speed (km/h) | 135 | 140 |
| 0–50 kph – 0–100 kph – 80–120 kph (s) | 3,9 – 11,4 – 9,3 | 3,6 – 9,5 – 7,1 |
| SCx | 0,75 |  |
| STEERING |  |  |
| Power | Yes (electric) |  |
| Turning circle between kerbs (m) | 10,56 |  |
| Number of steering-wheel turns | 2,73 |  |
| WHEELS AND TYRES |  |  |
| Standard wheel rims (") | 15’’ (Life) – 16’’ (Life, Zen) – 17’’ (Zen) | 16’’ (Zen, Intens) – 17’’ (Zen, Intens) |
| Tyre dimensions | Michelin Primacy 4 15’’: 185/65 R15 – 16’’: 195/55 R16 – 17’’: 215/45 R17 (front), 205/45 R17 (rear) | Michelin Primacy 4 16’’: 195/55 R16 – 17’’: 215/45 R17 (front), 205/45 R17 (rear) |
| BRAKING |  |  |
| Front: plain disc (PD), ventilated disc (VD) diam. (mm) | Ventilated disks, 280 |  |
| Rear: drum (D), plain disc (PD), ventilated disc (VD) diam. (mm) | Ventilated disks, 260 |  |
| WEIGHT (KG) |  |  |
| Unladen kerb weight | 1,502 |  |
| Maximum vehicle weight | 1,988 |  |
| Gross vehicle weight rating (GVWR) | 1,988 |  |
| Within the maximum permissible GVWR at front/rear | 1,048 / 940 |  |
| Payload (PL) (Life model, no options / Zen and Intens models, all options) | 425 / 474 | 425 / 486 |
| Max. braked/unbraked towed weight | Not allowed / Not allowed |  |

| BOOT VOLUME (ISO dm3) |  |  |
| --- | --- | --- |
|  | Under rear parcel shelf | 338 |
|  | Flat-folded rear bench seat, up to roof | 1,225 |
| DIMENSIONS (mm) |  |  |
| A | Wheelbase | 2,588 |
| B | Overall length | 4,084 |
| C | Front overhang | 838 |
| D | Rear overhang | 661 |
| E | Front track | 1,506 |
| F | Rear track | 1,489 |
| G | Overall width including / without mirrors | 1,730 / 1,945 |
| H | Unladen height | 1,562 |
| H1 | Unladen height with hatch open | 2,028 |
| J | Unladen sill height | 741 |
| K | Ground clearance (laden) | 120 |
| L | Cabin length | 1,678 |
| M | Front elbow room | 1,384 |
| M1 | Rear elbow room | 1,390 |
| N | Front shoulder room | 1,365 |
| N1 | Rear shoulder room | 1,320 |
| P | Front headroom measured at 14° | 904 |
| Q | Rear headroom measured at 14° | 826 |
| Y | Width at top of boot access | 941 |
| Y1 | Width at bottom of boot access | 899 |
| Y2 | Inside width between wheel arches | 1,037 |
| Z | Rear aperture heigth | 685 |
| Z1 | Maximum load length, flat-folded rear bench seat | 1,426 |
| Z2 | Heigth up to the parcel shelf | 622 |
| Z3 | Load length behind rear bench seat | 580 |
| HR01 | Rear seat cushion height | 298 |
| HR11 | Rearl knee radius | 157 |
| W6 | Width at hips second taken between arm tests | 1,247 |